| [Abstract] |  |  |  |  |  | KOČEVJE, SLOVENIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apt Elev 1765’/538m N45°34’12’’ E014°51’30’’ |  |  |  |  |  |  |  |  | 2021-07-15 |  |  |  | NOVI LAZI – KOČEVSKA REKA |  |  |  |  |  |  |  |
| AIR/GND |  |  |  | KOČEVJE AIRFIELD 123.505 |  |  |  |  |  |  | FIS |  |  |  |  | LJUBLJANA INFO 118.480 |  |  |  |  |
| Visual Approach Chart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RWY |  | DIMENSION |  |  |  | SURFACE |  | TORA |  |  | TODA |  |  |  | ASDA |  |  | LDA | LIGHTS | PATTERN |
| 15 |  | 300 x 30 m |  |  |  | grass |  | 300 |  |  | 300 |  |  |  | 300 |  |  | 300 | nil | LH & RH |
| 33 |  |  |  |  |  |  |  | 300 |  |  | 400 |  |  |  | 300 |  |  | 300 |  | LH & RH |
| NOTES: All the distances in meters. Not for official use. All data for information purposes only. Check latest info in official aerodrome manual and AIP before commencing any flight. Copyleft www.vfr-charts.ga |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [Abstract] |  |  |  |  |  |  | KOČEVJE, SLOVENIA |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apt Elev 1765’/538m N45°34’12’’ E014°51’30’’ |  |  |  |  |  |  | 2021-07-15 |  |  |  |  |  |  |  | NOVI LAZI – KOČEVSKA REKA |  |  |  |  |  |
| AIR/GND |  | KOČEVJE AIRFIELD 123.505 |  |  |  |  |  |  | FIS |  |  |  | LJUBLJANA INFO 118.480 |  |  |  |  |  |  |
| Ground Movement Chart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Warnings |  |  |  | - No continuous radio watch. Transmit blind. - Avoid flying over populated areas – villages Štalcarji and Novi Lazi. - Pay attention to the road before threshold 15. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks |  |  |  | - PPR – Prior permission required. - Only VFR flights allowed. - 100 m clearway marked with yellow flags located behind the departure end of runway 33 which is hardened and maintained the same way as main runway. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operator |  |  |  | Letalski klub Kočevje |  |  |  |  |  |  | Email |  |  |  |  | lk.kocevje@gmail.com |  |  |  |
| Address |  |  |  | Pri Unionu 16 |  |  |  |  |  |  | Internet |  |  |  |  | www.lk-kocevje.si |  |  |  |
| Zip City |  |  |  | SI-1330 Kočevje |  |  |  |  |  |  | ARO |  |  |  |  | +386 4 2040 420 – Ljubljana |  |  |  |
| Tel |  |  |  |  |  |  |  |  |  |  | METEO |  |  |  |  | +386 4 2804 500 – Ljubljana |  |  |  |
| Fax |  |  |  |  |  |  |  |  |  |  | Customs |  |  |  |  | N/A |  |  |  |
| Mobile |  |  |  | +386 41 430 745 |  |  |  |  |  |  | Fuel |  |  |  |  | N/A |  |  |  |

Not for official use. All data for information purposes only. Copyleft  www.vfr-charts.ga

| SYMBOL LEGEND |  |  |
| --- | --- | --- |
| Revision history |  |  |
| Revision | Date | Changes |
| v1 | 2021-07-15 | Initial version |